FM-1  MIDI

English Version

Channels: Note channel (midich) defaults to All / Omni; FX channel (effectch) defaults to MIDI ch 2; System Real-Time has no channel.

⚠ CC 1 (ModWheel) sent on the FX channel (default ch2) acts as an FX parameter (Filter Type), not ModWheel; send it on another channel. Pitch Bend is unaffected on any channel.

1. Note Channel (midich)

| Message | Byte | Data / CC | Response |
| --- | --- | --- | --- |
| Note On | 9n | note, vel 1–127 | Sound (vel 0 = Note Off) |
| Note Off | 8n | note | Key release |
| Program Change | Cn | 0–127 | Voice 001–128 |
| Pitch Bend | En | 14-bit | Pitch bend |
| Aftertouch | Dn | 0–127 | Channel aftertouch |
| Mod Wheel | Bn | CC 1 | Modulation |
| Sustain | Bn | CC 64 | Sustain (release on off) |

2. FX Channel (effectch, default ch2)  ·  CC 0–23

| CC | Effect | Parameter | Range |
| --- | --- | --- | --- |
| 0 | Filter | Switch | 0=off / ≥1=on |
| 1 | Filter | Type | 0–2 (LPF/BPF/HPF) |
| 2 | Filter | Cutoff | 0–107 |
| 3 | Filter | Q | 0–10 |
| 4 | Reverb | Switch | 0=off / ≥1=on |
| 5 | Reverb | Type | 0–2 (Room/Hall/Plate) |
| 6 | Reverb | Decay | 0–100 |
| 7 | Reverb | Mix | 0–100 |
| 8 | Delay | Switch | 0=off / ≥1=on |
| 9 | Delay | Decay | 0–100 |
| 10 | Delay | Rate | 0–100 |
| 11 | Delay | Mix | 0–100 |
| 12 | Distortion | Switch | 0=off / ≥1=on |
| 13 | Distortion | Gain | 0–100 |
| 14 | Distortion | Tone | 0–100 |
| 15 | Distortion | Level | 0–100 |
| 16 | Chorus | Switch | 0=off / ≥1=on |
| 17 | Chorus | Freq | 0–100 |
| 18 | Chorus | Depth | 0–100 |
| 19 | Chorus | Mix | 0–100 |
| 20 | Phaser | Switch | 0=off / ≥1=on |
| 21 | Phaser | Freq | 0–100 |
| 22 | Phaser | Depth | 0–100 |
| 23 | Phaser | Mix | 0–100 |

3. System Real-Time (no channel)

| Byte | Message | Response |
| --- | --- | --- |
| F8 | Clock | Follow external BPM |
| FA | Start | Start from step 0 |
| FB | Continue | Same as Start (no resume) |
| FC | Stop | Stop |

SysEx: F0 43 10 pp qq vv F7 → single-parameter write (param = pp×128+qq, 0–155; value vv).