MIDI CC Control Mapping Table

Conventions

B0 is the Control Change header for MIDI Channel 1. Send 127 for "ON / Execute", 0 for "OFF". Mid-value 64 is the threshold: ≥64 = ON, <64 = OFF.

I. Drum Machine (2 functions)

| CC | Function | ON (Execute) | OFF |
| --- | --- | --- | --- |
| 22 | Drum Play/Stop | 64–127 | 0–63 |
| 23 | Drum BPM Follow | 64–127 | 0–63 |

II. Looper (9 functions)

(A) Basic Controls

| CC | Function | Value Mapping | Description |
| --- | --- | --- | --- |
| 21 | Looper Toggle / Stop | 127 / 0 | Send 127: cycles through Record → Play → Record (overdub); Send 0: Stop |
| 32 | Looper Stop | 127 | Immediate stop |
| 33 | Looper Undo / Clear | 127 | Undo last overdub / Clear entire loop |
| 36 | Loop Point Toggle (Head/Tail) | 127 (Tail) / 0 (Head) | Toggle between looping back to head or stopping at tail |

(B) Auto-Record Modes

| CC | Function | ON | OFF |
| --- | --- | --- | --- |
| 34 | Auto-Record – One-Shot Mode | 127 | 0 |
| 35 | Auto-Record – Hold Mode | 127 | 0 |

These two CCs share a single Auto-Record state. Sending 34=127 switches to One-Shot; sending 35=127 switches to Hold; sending either with 0 turns Auto-Record OFF.

(C) Drum Sync

| CC | Function | ON | OFF |
| --- | --- | --- | --- |
| 37 | Drum Sync Mode 1 | 127 | 0 |
| 39 | Drum Sync Mode 2 | 127 | 0 |

III. New 12 CCs – Detailed Table

(A) Button Emulation Group (CC 50–57, 8 functions)

Acts like remote physical footswitch/button commands.
Trigger rule: Send val ≥ 64 to trigger.

| CC | Function | Value Mapping |
| --- | --- | --- |
| 50 | FS1 – Short Press | ≥ 64 |
| 51 | FS2 – Short Press | ≥ 64 |
| 52 | CtrlA – Short Press | ≥ 64 |
| 53 | CtrlB – Short Press | ≥ 64 |
| 54 | FS1 – Long Press | ≥ 64 |
| 55 | FS2 – Long Press | ≥ 64 |
| 56 | CtrlA – Long Press | ≥ 64 |
| 57 | CtrlB – Long Press | ≥ 64 |

(B) System / Audio Parameters (CC 58–61, 4 functions)

Continuous values 0–127 are mapped via usr_map to the target parameter range.

| CC | Function | Value Range | Description |
| --- | --- | --- | --- |
| 58 | RCH – Right Channel Output Mode | 0–2 | 0 = Disable / 1 = Enable / 2 = Dry |
| 59 | USB Audio Type | 0–3 | 0 = Nor / 1 = No / 2 = Resample / 3 = Dry |
| 60 | BT & USB Playback Volume | 0–100 | Bluetooth / USB playback volume |
| 61 | USB Rec Vol | 0–100 | USB recording return volume (50 = 0dB unity) |